# IPv4 Addressing and Networks 816032904

IPv4 Addresses on interfaces for routers facing other routers.

| Name | Interface | VLAN | IP Address | Subnet Mask | Network Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- |
| PC11 | Eth0 | 81 | 192.32.90.11 | 255.255.255.128 | 192.32.90.0/25 | 192.32.90.127 |
| PC12 | Eth0 | 81 | 192.32.90.12 | 255.255.255.128 | 192.32.90.0/25 | 192.32.90.127 |
| PC13 | Eth0 | 81 | 192.32.90.13 | 255.255.255.128 | 192.32.90.0/25 | 192.32.90.127 |
| PC14 | Eth0 | 81 | 192. 32.90.14 | 255.255.255. 128 | 192.32.90.0/25 | 192.32.90.127 |
| PC21 | Eth0 | 111 | 192.32.90.131 | 255.255.255.128 | 192.32.90.128/25 | 192.32.90.255 |
| PC22 | Eth0 | 111 | 192.32.90.132 | 255.255.255.128 | 192.32.90.128/25 | 192.32.90.255 |
| PC23 | Eth0 | 111 | 192.32.90.133 | 255.255.255.128 | 192.32.90.128/25 | 192.32.90.255 |
| PC24 | Eth0 | 111 | 192.32.90.134 | 255.255.255.128 | 192.32.90.128/25 | 192.32.90.255 |
| PC25 | Eth0 | 111 | 192.32.90.135 | 255.255.255.128 | 192.32.90.128/25 | 192.32.90.255 |
| PC31 | Eth0 | 11 | 192. 32.91.31 | 255.255.255.128 | 192. 32.91.0/25 | 192. 32.91.127 |
| PC32 | Eth0 | 11 | 192. 32.91.32 | 255.255.255.128 | 192. 32.91.0/25 | 192. 32.91.127 |
| PC33 | Eth0 | 11 | 192. 32.91.33 | 255.255.255.128 | 192. 32.91.0/25 | 192. 32.91.127 |
| Server21 | Eth0 | 5 | 192.32.91.221 | 255.255.255.128 | 192.32.91.128/25 | 192. 32.91.255 |
| Server11 | Eth0 | 5 | 192.32.91.211 | 255.255.255.128 | 192.32.91.128/25 | 192. 32.91.255 |

# IPv6 Addressing and Networks

IPv6 Addresses on interfaces for routers facing other routers.

| Name | Interface | VLAN | IP v6 Address | Prefix |
| --- | --- | --- | --- | --- |
| PC11 | Eth0 | 81 | 2001:CAFE:8160:3290::4011/ 64 | 2001:CAFE:8160:3290::/64 |
| PC12 | Eth0 | 81 | 2001:CAFE:8160:3290::4012/ 64 | 2001:CAFE:8160:3290::/64 |
| PC13 | Eth0 | 81 | 2001:CAFE:8160:3290::4013/ 64 | 2001:CAFE:8160:3290::/64 |
| PC14 | Eth0 | 81 | 2001:CAFE:8160:3290::4014/ 64 | 2001:CAFE:8160:3290::/64 |
| PC21 | Eth0 | 111 | 2001:CAFE:8160:3290::4021/ 64 | 2001:CAFE:8160:3290::/64 |
| PC22 | Eth0 | 111 | 2001:CAFE:8160:3290::4022/ 64 | 2001:CAFE:8160:3290::/64 |
| PC23 | Eth0 | 111 | 2001:CAFE:8160:3290::4023/ 64 | 2001:CAFE:8160:3290::/64 |
| PC24 | Eth0 | 111 | 2001:CAFE:8160:3290::4024/ 64 | 2001:CAFE:8160:3290::/64 |
| PC25 | Eth0 | 111 | 2001:CAFE:8160:3290::4025/ 64 | 2001:CAFE:8160:3290::/64 |
| PC31 | Eth0 | 11 | 2001:CAFE:8160:3290::4031/ 64 | 2001:CAFE:8160:3290::/64 |
| PC32 | Eth0 | 11 | 2001:CAFE:8160:3290::4032/ 64 | 2001:CAFE:8160:3290::/64 |
| PC33 | Eth0 | 11 | 2001:CAFE:8160:3290::4033/ 64 | 2001:CAFE:8160:3290::/64 |
| Server11 | Eth0 | 5 | 2001:CAFE:8160:3290::4044/ 64 | 2001:CAFE:8160:3290::/64 |
| Server21 | Eth0 | 5 | 2001:CAFE:8160:3290::4045/ 64 | 2001:CAFE:8160:3290::/64 |

  - 10) From PC11 ping the IPv4 address of the following, record commands used AND screenshots of the ping operation,

  - i) PC12

pc11> ping 192.32.90.12

  - ii) PC14

  - pc11>ping 192.32.90.14

  - (I only got this ping to work by temporarily assigning port 7 on switch 11 vlan 111 when it supposed to be vlan 81. All other pings work as they should without temporarily changing anything.)

  - 11) From PC21 ping the IPv4 address of the following, record commands used AND screenshots of the ping operation,

  - i) PC22

  - ii) PC25

  - 12) From PC21 ping the IPv6 address of the following, record commands used AND screenshots of the ping operation,

  - i) PC24

  - PC21>ping 2001:CAFE:8160:3290::4024 -w 5000

  - ii) PC25

  - PC21> ping 2001:CAFE:8160:3290::4025 -w 5000

  - 13) From PC31 ping the IPv6 address of the following, record commands used AND screenshots of the ping operation,

  - i) PC32

  - PC31> ping 2001:CAFE:8160:3290::4032 -w 5000

  - ii) PC33

  - PC31> ping 2001:CAFE:8160:3290::4033 -w 5000

Commands Used

[used this as an example to place your commands in your work document]

PC11

ip 192.32.90.11 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4011/64 auto

show ipv6

save

PC12

ip 192.32.90.12 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4012/64 auto

show ipv6

save

PC 13

ip 192.32.90.13 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4013/64 auto

show ipv6

save

PC 14

ip 192.32.90.14 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4014/64 auto

show ipv6

save

PC 21

ip 192.32.90.131 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4021/64 auto

show ipv6

save

PC 22

ip 192.32.90.132 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4022/64 auto

show ipv6

save

PC 23

ip 192.32.90.133 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4023/64 auto

show ipv6

save

PC 24

ip 192.32.90.134 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4024/64 auto

show ipv6

save

PC25

ip 192.32.90.135 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4025/64 auto

show ipv6

save

PC31

ip 192.32.91.31 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4031/64 auto

show ipv6

save

PC32

ip 192.32.91.32 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4032/64 auto

show ipv6

save

PC33

ip 192.32.91.33 255.255.255.128

show ip

ip 2001:CAFE:8160:3290::4033/64 auto

show ipv6

save

Switch configuration:

SW12

En

Show vlan-switch brief

Vlan database

Vlan 81 Name Student

Vlan 11 Name Accounts

Vlan 111 Name Research

Vlan 5 Name Server

Vlan 999 Name BlackHole

Exit

En

Show vlan-switch brief

Conf t

Int range Fa1/0 - 1

Switchport access vlan 81

Switchport mode access

Shutdown

No Shutdown

end

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface Fa1/12

Switchport access vlan 11

Switchport mode access

Shutdown 
no shut

End

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface Fa1/11

Switchport access vlan 5

Switchport mode access

Shutdown

No shut

End

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface range fa1/2 - 10

Switchport access vlan 999

Switchport mode access

Shutdown

No shut

End

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface range Fa2/0-15

Switchport access vlan 999

Switchport mode access

Shutdown

No shut

End

Show vlan-switch brief

Setting up trunk ports.

Show vlan

Conf t

Int fa1/11

Switchport mode trunk

switchport trunk encapsulation dot1q

Shutdown

No shutdown

End

Show vlan

Show vlan

Conf t

Int fa1/13

Switchport mode trunk

switchport trunk encapsulation dot1q

Shutdown

No shutdown

End

Show vlan

En

Show int trunk

Conf term

Interface range Fa1/14 - 15

Switchport trunk

switchport trunk encapsulation dot1q

Shutdown

No shutdown

End

Show vlan

Show vlan

show interfaces trunk

Conf t

Interface Fa1/11

switchport trunk allowed vlan remove 999

end

Show interfaces trunk

Show vlan

show interfaces trunk

Conf t

Interface Fa1/13

switchport trunk allowed vlan remove 999

end

show interfaces trunk

En

show interfaces trunk

Conf t

Interface range Fa1/14 - 15

Switchport trunk allowed vlan remove 999

End

show interfaces trunk

Write memory

Switch 22

En

Vlan database

Vlan 111 name Research

Vlan 5 name Server

Vlan 999 name BlackHole

Exit

Show vlan-switch brief

Conf t

interface range Fa1/0 – 1

Switchport access vlan 111

Switchport mode access

Shutdown

No Shutdown

end

En

Show vlan-switch brief

Conf t

Interface Fa1/12

Switchport access vlan 5

Switchport mode access

Shutdown 
no shut

End

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface range Fa1/2 - 12

Switchport access vlan 999

Switchport mode access

Shutdown

No shut

End

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface Fa1/15

Switchport access vlan 999

Switchport mode access

Shutdown

No shut

End

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface range Fa2/0 - 15

Switchport access vlan 999

Switchport mode access

Shutdown

No shut

End

Show vlan-switch brief

En

Show vlan-switch brief

Configure t

Interface 1/13

Switchport access vlan 5

Switchport mode access

Shutdown

No shutdown

End

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface  Fa1/14

Switchport mode trunk

switchport trunk encapsulation dot1q

Shutdown

No shutdown

End

Show vlan-switch brief

En

Show int trunk

Conf term

Interface range Fa1/14

Switchport trunk allowed vlan remove 999

End

Show int trunk

Write memory

SW21

En

Vlan database

Vlan 111 name research

Vlan 5 Name Server

Vlan 999 Name BlackHole

Exit

En

Show vlan-switch brief

Conf t

Int range Fa1/0 - 1

Switchport access vlan 111

Switchport mode access

Shutdown

No Shutdown

end

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface range Fa1/2-13

Switchport access vlan 999

Switchport mode access

Shutdown

No shut

End

Show vlan-switch brief

En

Show vlan-switch brief

Conf t

Interface range fa2/0-15

Switchport access vlan 999

Switchport mode access

Shutdown

No shut

End

Show vlan-switch brief

Setting up trunk ports.

Show vlan-switch brief

Conf t

Interface range  Fa1/14 to 15

Switchport mode trunk

switchport trunk encapsulation dot1q

Shutdown

No shutdown

End

Show vlan-switch brief

En

Show int trunk

Conf term

Interface range Fa1/14 to 15

Switchport trunk allowed vlan remove 999

End

Show int trunk

Write memory