Group 14:

Project Report on Creating a Fully Functional LAN on GNS3

INFO 3605 - Fundamentals of LAN Technologies

Group Members:

Ronel Ramtahal – 816017853 – ronel.ramtahal@my.uwi.edu

Jonathan Trim – 816032904 - jonathan.trim@my.uwi.edu

Andrea Birjah-Rampersad – 816035169 – andrea.birjahrampersad@my.uwi.edu

Kenwyn Simon – 816037264 - kenwyn.simon@my.uwi.edu

Initial Version: 16.11.24

Updated Version: 30.11.24

# Table of Contents

1 Executive Summary	3

2 Introduction	4

3 Literature Review	5

4 Proposal	9

5 Methodology	10

6 Execution, Testing and Results	13

7 Summary, Issues and Limitations	16

8 Conclusion	18

9 Appendix I - Output

10 Appendix II - References

# Executive Summary

The goal of this project aimed to utilize GNS3 to build and configure a fully functioning Local Area Network (LAN) using IP addressing schemes, VLANs, EtherChannel, and various routing protocols (RIP, OSPF, and EIGRP). Through the implementation of routing protocol redistribution and network performance efficiency, the objective was to preserve smooth communication across various network parts.

Planning included understanding the assignment specifications, assigning responsibilities according to each persons strengths as well as making use of resources from online resources, labs and lectures. We worked together to test VLANs and EtherChannel settings, establish routing protocols and design the network topology.

We successfully tested routing redistribution, set up VLANs, defined IP addressing, and implemented the necessary routing protocols. By means of detailed troubleshooting and configuration, we made sure that every device on the network could successfully interact with each other. 
With all network segments linked and the routing protocols operating as planned, the project was successfully finished, showcasing efficient teamwork and problem-solving.

# Introduction

The project's main goal is to create a dependable and flexible network for a fictional company called Tech Haven, which specializes in software development and IT services. The three primary departments that make up the company's infrastructure—Development, Operations, and Support—all need effective departmental and interdepartmental communication. A high-performance Local Area Network (LAN) that facilitates secure data transmission, efficient traffic flow and interdepartmental communication is required by the environment.

Four committed members of our project greoup, each with a unique set of networking and system design talents, pooled their knowledge to take on this difficult challenge. Each member brought special talents to the table, such as practical knowledge of IP addressing, routing protocols, and network simulation programs like GNS3. Throughout the project, efficient planning and execution was ensured by collaboration.
Key requirements for the project were setting up VLANs for departmental segmentation, EtherChannel for bandwidth optimization, Spanning Tree Protocol (STP) for loop prevention, and routing protocols (RIP, OSPF, and EIGRP). To guarantee smooth communication between every device on the network, the effort also involved creating an effective IP addressing system and setting redistribution of the routing protocol into order.

# Literature Review

Technologies Implemented:

This project integrates several foundational and advanced networking technologies to create a functional and scalable Local Area Network (LAN) inclusive of:

- Routing Protocols:

  - RIP (Routing Information Protocol): A distance-vector protocol that utilizes the hop count to select routes. Although it works well for local networks, its limited scalability (maximum of 15 hops) and delayed convergence make it ineffective for larger networks.

  - OSPF (Open Shortest Path First): A link-state protocol that determines the shortest route to a destination using Dijkstra's algorithm. With its quick convergence and hierarchical routing, it works well for bigger, more complicated networks. It is resource-intensive, though, requiring a large amount of memory and processing power.

  - EIGRP (Enhanced Interior Gateway Routing Protocol): A Cisco-exclusive protocol combines the most significant aspects of link-state and distance-vector protocols. It enables load balancing, offers fast convergence, and is scalable. Its proprietary nature, which limits compatibility with non-Cisco equipment, is its main drawback.

- VLANs (Virtual Local Area Networks):
By allowing networks to be logically segmented, VLANs increase security and decrease broadcast traffic. They enable communication between devices in the same VLAN even if they are located in separate places, just like if they were on the same physical network. Though careful planning is necessary to prevent broadcast storms and misconfigurations, proper VLAN installation guarantees effective traffic management.

- EtherChannel:
EtherChannel provides redundancy and enhanced capacity by combining several physical channels into a single logical link. Although this technology lessens network congestion, troubleshooting may become more difficult because both ends must have the same specifications.

- Spanning Tree Protocol (STP):
By establishing a spanning tree topology, STP stops loops in Ethernet LANs. It blocks redundant paths and guarantees that there is only one active path between network devices. STP can introduce latency during its convergence process, although being necessary for loop prevention..

- Routing Redistribution:
To facilitate smooth communication between segments utilizing various routing protocols, redistribution is put into place. All devices in the network can communicate regardless of the protocol used in their individual segments because to the exchange of route information across protocols..

Tools & Resources Used:

- GNS3:

  - Utilized as the main simulation platform for network topology design and setup, routing protocol testing and configuration troubleshooting.

- IP Addressing and Subnetting:

  - VLSM (Variable Length Subnet Mask) was employed to maximize IP address utilization, particularly in Segment 3, whereas Class C IP addresses were utilized for convenience. For precise subnet planning, tools such as the IP Subnet Calculator were employed.

- Cisco IOS Commands:

  - VLANs, EtherChannel, Spanning Tree Protocol, and routing protocols were configured using Cisco equipment and their IOS (Internetwork Operating System) instructions.

- Online Tutorials and Documentation:

  - MustBeGeek: Provided step-by-step guides on configuring routing protocols and redistribution.

  - NetworkLessons: Offered detailed tutorials on VLAN creation, EtherChannel configuration and routing protocol setup.

  - Cisco documentation was used as an authoritative source for implementing VLANs and troubleshooting network configurations.

- Collaborative Tools:

  - Tools like Google Docs, WhatsApp, Discord and the LMS forums were instrumental for planning and team collaboration.

# Proposal

In order to resolve Tech Haven's networking requirements, we suggest creating a scalable and effective Local Area Network (LAN) that guarantees smooth communication, enhanced performance, and security for every department.

Three primary network segments, each running a different routing protocol (RIP, OSPF, and EIGRP), will make up the network's hierarchical architecture. To allow communication between these portions, routing redistribution will be set up. To improve security and lessen congestion, departmental traffic will be divided using VLANs.

We'll use Spanning Tree Protocol (STP) to avoid network loops and EtherChannel to boost bandwidth and offer link redundancy. An organized IP addressing system that makes use of VLSM and private IP ranges will guarantee effective address distribution.

To confirm functionality and make sure the network can support the business's activities and expand with it, all configurations will be evaluated in GNS3. This solution offers a reliable, effective and safe network architecture that is customized to meet Tech Haven's requirements.

# Methodology

Plan of Action:

Action Step 1: Analyze Project Requirements

- Review the project objectives and requirements, including routing protocols (RIP, OSPF, EIGRP), VLAN configuration, IP addressing & routing redistribution.

- Understand the dependencies between each component and plan their integration.

Action Step 2: Design the Network Topology

- Create a detailed network diagram using GNS3 to map out the routers, switches, and end devices.

- Label all devices and interfaces for clarity.

- Include the configuration of VLANs and their associated switches in the diagram.

Action Step 3: Develop an IP Addressing Scheme

- Design a structured IP addressing plan, using private IP ranges and VLSM for efficient subnetting in Segment 3.

- Prepare a network table to document: device names, interface IP addresses, subnet masks, gateway addresses.

Action Step 4: Configure Routing Protocols and VLANs

- Configure each segment with its assigned routing protocol (RIP, OSPF, EIGRP).

- Set up VLANs on switches and assign IP subnets to each VLAN.

- Verify VLAN tagging and inter-VLAN routing functionality.

Action Step 5: Implement Routing Redistribution

- Configure redistribution between RIP, OSPF, and EIGRP on routers where segments overlap.

- Test and verify that all devices can communicate across the network.

Action Step 6: Configure EtherChannel and STP

- Set up EtherChannel to aggregate links between switches, enhancing bandwidth and redundancy.

- Enable Spanning Tree Protocol (STP) to prevent loops and designate the root switch.

Action Step 7: Test Network Functionality

- Perform connectivity tests using pings and traceroutes to ensure communication between devices.

- Verify routing adjacencies, VLAN functionality & redundancy configurations.

Action Step 8: Document and Present

- Compile all configurations, test results & screenshots into a report.

- Prepare a presentation to showcase the network design, configurations, and project results.

# Execution, Testing and Results

In GNS3, the project started with the development of the network design. Three segments were created, each of which was using a different routing protocol: RIP for Segment 1, OSPF for Segment 2, and EIGRP for Segment 3. In accordance with the project specifications, every device—including PCs, switches, and routers—was connected and labeled. Using private Class C ranges, a structured IP addressing scheme was created using our student IDs and Segment 3 was subnetted effectively by using VLSM. Each VLAN has its own subnet assigned to it, preventing IP range overlaps.

Using the proper RIP, OSPF, and EIGRP commands, adjacency development was confirmed and routing protocols were set up for their corresponding segments. After creating and assigning VLANs (311, 321, 331, and 341) to their corresponding switches, routers were configured with inter-VLAN routing to allow VLANs to communicate with one another. At routers that overlapped, routing redistribution between the protocols was then put into place. Careful adjustments were made to the metrics to guarantee appropriate route sharing and avoid routing loops.

To provide more bandwidth and reliability, EtherChannel was set up to combine multiple connections across switches. For a topology without loops, Spanning Tree Protocol (STP) was activated, and one switch was manually configured as the root bridge for the best path selection. Using the ping commands, the whole network was thoroughly examined to confirm successful connections across all segments and VLANs and traceroutes were used to show the sequence of packets from one device to another. The process of debugging revealed and fixed issues including IP overlaps, redistribution metrics, and erroneous VLAN tagging.

The final outcome was a fully functional network with smooth communication across all segments and VLANs for all devices. All protocols were connected thanks to routing redistribution, and EtherChannel's STP enhanced redundancy and speed. With a carefully thought-out IP addressing scheme that permits future growth, the network was built to be both safe and scalable. Appendix I contains screenshots that support the project report's documentation of all setups, outputs, and test results. The effective accomplishment of all project goals was guaranteed by this methodical implementation.

# Summary, Issues and Limitations

We encountered difficulties with group communication, the GNS3 simulation environment, and routing protocols throughout the project. At first, we thought that implementing protocols like OSPF, EIGRP, and RIP would be simple. However, even though the right instructions were used, initial efforts resulted in failed pings. This demonstrated the necessity of progressive testing and thorough debugging. Additionally, certain commands that were successful in Cisco Packet Tracer did not function in GNS3, indicating platform-specific variations. IOS-specific commands and modifications were needed for GNS3, therefore previous study and tool familiarity were crucial.

Another issue were GNS3's default settings, as some protocol setups failed without modifying the types of interfaces or router resources. Because some group members' computers did not have the 8–16 GB of RAM that GNS3 recommends, running numerous routing protocols and VLANs at once put a load on the devices. Progress was put on hold and simulations had to be resumed as a result of slower simulations and occasional failures.

Due to individual responsibilities, it was challenging to coordinate the schedules of group members, which created extra challenges for collaboration. Collaborative work was slowed down by the inadequacy of WhatsApp as a communication tool for exchanging huge files and intricate designs. Despite these obstacles, the team was able to adjust by using more tools as needed, remaining persistent and debugging.

In conclusion, we used RIP, OSPF, and EIGRP to effectively design and deploy a working Local Area Network (LAN) over three segments. For logical traffic segmentation, VLANs were given to a structured IP addressing scheme that included VLSM for Segment 3. While EtherChannel and Spanning Tree Protocol (STP) were used to increase capacity and avoid network loops, routing redistribution made sure that communication across segments ran smoothly. Using the ping and traceroute commands, connectivity was thoroughly examined to ensure that all project specifications were satisfied. Upcoming projects may learn a lot from these difficulties and results, which highlight the significance of planning, resource management, and productive teamwork.

# Conclusion

This project successfully implemented a functional LAN using RIP, OSPF and EIGRP, with routing redistribution ensuring seamless communication across segments. VLANs, EtherChannel, and STP were configured, and all requirements were met after rigorous testing. However, challenges such as GNS3 crashes, slower simulations on underpowered laptops & coordination issues among group members arose. These were overcome through troubleshooting, using alternative devices & improving collaboration tools like Google Drive.

For future projects, it’s recommended to use higher-performance devices to avoid simulation delays, consider alternative software like Cisco Packet Tracer and plan subnetting and routing in smaller setups first. Better collaboration platforms, more time for testing & real-time documentation will improve efficiency and ensure smoother execution. Addressing these areas will lead to even more successful outcomes in future projects.

# Appendix I - Output

| Name | Interface | IPv4 | Subnet Mask | Net Add | Broadcast | Gateway | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SEGMENT X (SEGMENT 1) |  |  |  |  |  |  |  |
| X1 | f0/1 | 192.168.78.145 | 255.255.255.240 | 192.168.78.144 | 192.168.78.159 | - | P2P Y11X |
|  | f0/0 | 192.168.78.1 | 255.255.255.240 | 192.168.78.0 | 192.168.78.0 | - | P2P X2 |
|  | f1/0 | 192.168.53.1 | 255.255.255.224 | 192.168.53.0 | 192.168.53.31 |  | P2P X9Z |
| X2 | f0/1 | 192.168.78.2 | 255.255.255.240 | 192.168.78.0 | 192.168.78.15 |  | P2P to X1 |
|  | f0/0 | 192.168.78.17 | 255.255.255.240 | 192.168.78.16 | 192.168.78.15 |  | P2P to X3 |
|  | f1/0 | 192.168.53.33 | 255.255.255.224 | 192.168.53.32 | 192.168.53.63 |  | P2P to X8Z |
| X3 | f0/1 | 192.168.78.18 | 255.255.255.240 | 192.168.78.16 | 192.168.78.16 |  | P2P to X2 |
|  | f0/0 | 192.168.78.33 | 255.255.255.240 | 192.168.78.32 | 192.168.78.47 |  | P2P to X4 |
|  | f1/0 | 192.168.53.65 | 255.255.255.224 | 192.168.53.64 | 192.168.53.64 |  | P2P to X7 |
| X4 | f0/1 | 192.168.78.34 | 255.255.255.240 | 192.168.78.32 | 192.168.78.47 |  | P2P to X3 |
|  | f0/0 | 192.168.78.49 | 255.255.255.240 | 192.168.78.48 | 192.168.78.63 |  | P2P to X5 |
| X5 | f0/1 | 192.168.78.50 | 255.255.255.240 | 192.168.78.48 | 192.168.78.63 |  | P2P to X4 |
|  | f0/0 | 192.168.78.65 | 255.255.255.240 | 192.168.78.64 | 192.168.78.79 |  | P2P to X6 |
| X6 | f0/1 | 192.168.78.66 | 255.255.255.240 | 192.168.78.64 | 192.168.78.79 |  | P2P to X5 |
|  | f0/0 | 192.168.78.81 | 255.255.255.240 | 192.168.78.80 | 192.168.78.95 |  | P2P to X7 |
| X7 | f0/1 | 192.168.78.82 | 255.255.255.240 | 192.168.78.80 | 192.168.78.95 |  | P2P to X6 |
|  | f0/0 | 192.168.78.97 | 255.255.255.240 | 192.168.78.96 | 192.168.78.111 |  | P2P to X8 |
|  | f1/0 | 192.168.53.66 | 255.255.255.224 | 192.168.53.64 | 192.168.53.95 |  | P2P to X3 |
| X8Z | f0/1 | 192.168.78.98 | 255.255.255.240 | 192.168.78.96 | 192.168.78.111 |  | P2P to X7 |
|  | f0/0 | 192.168.78.113 | 255.255.255.240 | 192.168.78.112 | 192.168.78.127 |  | P2P to X9Z |
|  | f1/0 | 192.168.53.2 | 255.255.255.224 | 192.168.53.0 | 192.168.53.31 |  | P2P to X2 |
|  | f3/0 |  |  |  |  |  | P2P To ZESW4X |
| X9Z | f0/1 | 192.168.78.114 | 255.255.255.240 | 192.168.78.112 | 192.168.78.12 |  | P2P to X8Z |
|  | f0/0 | 192.168.78.129 | 255.255.255.240 | 192.168.78.128 | 192.168.78.143 |  | P2P to Y10XZ |
|  | f1/0 | 192.168.53.2 | 255.255.255.224 | 192.168.53.0 | 192.168.53.31 |  | P2P to X1 |
|  | f4/0 |  |  |  |  |  | P2P To ZESW4X |
| Y1 | f0/1 | 192.168.29.1 | 255.255.255.240 | 192.168.29.0 | 192.168.29.15 |  | P2P Y2 |
|  | f0/0 | 192.168.29.145 | 255.255.255.240 | 192.168.29.144 | 192.168.29.159 |  | P2P Y11X |
|  | f1/0 | 192.168.4.33 | 255.255.255.224 | 192.168.4.32 | 192.168.4.63 |  | P2P Y9Z |
| Y2 | f0/1 | 192.168.29.17 | 255.255.255.240 | 192.168.29.16 | 192.168.29.16 |  | P2P Y3 |
|  | f0/0 | 192.168.29.2 | 255.255.255.240 | 192.168.29.0 | 192.168.29.15 |  | P2P Y1 |
|  | f1/0 | 192.168.4.65 | 255.255.255.244 | 192.168.4.64 | 192.168.4.107 |  | P2P Y8 |
| Y3 | f0/1 | 192.168.29.33 | 255.255.255.240 | 192.168.29.32 | 192.168.29.47 |  | P2P Y4 |
|  | f0/0 | 192.168.29.18 | 255.255.255.240 | 192.168.29.16 | 192.168.29.31 |  | P2P Y2 |
|  | f1/0 | 192.168.4.97 | 255.255.255.244 | 192.168.4.96 | 192.168.4.107 |  | P2P Y7 |
| Y4 | f0/1 | 192.168.29.49 | 255.255.255.240 | 192.168.29.48 | 192.168.29.63 |  | P2P Y4 |
|  | f0/0 | 192.168.29.34 | 255.255.255.240 | 192.168.29.32 | 192.168.29.47 |  | P2P Y3 |
|  | f1/0 | 192.168.4.129 | 255.255.255.244 | 192.168.4.128 | 192.168.4.139 |  | P2P Y6 |
| Y5 | f0/0 | 192.168.29.50 | 255.255.255.244 | 192.168.29.48 | 192.168.29.51 |  |  |
|  | f2/0 | 192.168.29.65 | 255.255.255.240 | 192.168.29.64 | 192.168.29.79 |  |  |
| Y6 | f0/1 | 192.168.29.81 | 255.255.255.240 | 192.168.29.80 | 192.168.29.95 |  |  |
|  | f0/0 | 192.168.29.66 | 255.255.255.240 | 192.168.29.64 | 192.168.29.79 |  |  |
|  | f1/0 | 192.168.4.130 | 255.255.255.244 | 192.168.4.128 | 192.168.4.128 |  |  |
| Y7 | f0/1 | 192.168.29.97 | 255.255.255.240 | 192.168.29.96 | 192.168.29.111 |  |  |
|  | f0/0 | 192.168.29.82 | 255.255.255.240 | 192.168.29.80 | 192.168.29.95 |  |  |
|  | f1/0 | 192.168.4.198 | 255.255.255.244 | 192.168.4.196 | 192.168.4.207 |  |  |
| Y8 | f0/1 | 192.168.29.113 | 255.255.255.240 | 192.168.29.112 | 192.168.29.127 |  |  |
|  | f0/0 | 192.168.29.98 | 255.255.255.240 | 192.168.29.96 | 192.168.29.111 |  |  |
|  | f1/0 | 192.168.4.66 | 255.255.255.244 | 192.168.4.64 | 192.168.4.75 |  |  |
| Y9Z | f0/1 | 192.168.29.129 | 255.255.255.240 | 192.168.29.128 | 192.168.29.143 |  |  |
|  | f0/0 | 192.168.29.114 | 255.255.255.240 | 192.168.29.112 | 192.168.29.127 |  |  |
|  | f1/0 | 192.168.4.34 | 255.255.255.244 | 192.168.4.32 | 192.168.4.43 |  |  |
| Y10XZ | f0/1 | 192.168.78.130 | 255.255.255.240 | 192.168.78.128 | 192.168.78.143 |  |  |
|  | f0/0 | 192.168.29.130 | 255.255.255.240 | 192.168.29.128 | 192.168.29.143 |  |  |
|  | f1/0 | 192.168.4.2 | 255.255.255.244 | 192.168.4.0 | 192.168.4.11 |  |  |
| Y11X | f0/1 | 192.168.29.146 | 255.255.255.240 | 192.168.29.144 | 192.168.29.159 |  |  |
|  | f0/0 | 192.168.78.146 | 255.255.255.240 | 192.168.78.144 | 192.168.78.159 |  |  |
|  | f1/0 | 192.168.4.1 | 255.255.255.244 | 192.168.4.0 | 192.168.4.11 |  |  |

| Device Name | Int/sub-Int | VLAN | IPv4 | Subnet Mask | Network Address | Broadcast Address | Gateway |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X1 | F2/0.10 | 10 | 10.1.0.1 | 255.255.255.224 | 10.1.0.0 | 10.1.0.31 |  |
|  | F2/0.20 | 20 | 10.2.0.1 | 255.255.255.224 | 10.2.0.0 | 10.2.0.31 |  |
|  | F2/0.30 | 30 | 10.3.0.1 | 255.255.255.224 | 10.3.0.0 | 10.3.0.31 |  |
| AX1 | e0 | 10 | 10.1.0.2 | 255.255.255.224 | 10.1.0.0 | 10.1.0.31 | 10.1.0.1 |
| BX1 | e0 | 20 | 10.2.0.2 | 255.255.255.224 | 10.2.0.0 | 10.2.0.31 | 10.2.0.1 |
| CX1 | e0 | 30 | 10.3.0.2 | 255.255.255.224 | 10.3.0.0 | 10.3.0.31 | 10.3.0.1 |
| X2 | f2/0.30 | 10 | 10.1.0.33 | 255.255.255.224 | 10.1.0.0 | 10.1.0.31 |  |
|  | f2/0.30 | 20 | 10.2.0.33 | 255.255.255.224 | 10.2.0.0 | 10.2.0.31 |  |
|  | f2/0.30 | 30 | 10.3.0.33 | 255.255.255.224 | 10.3.0.0 | 10.3.0.31 |  |
| AX2 | e0 | 10 | 10.1.0.34 | 255.255.255.224 | 10.1.0.0 | 10.1.0.31 | 10.1.0.33 |
| BX2 | e0 | 20 | 10.2.0.34 | 255.255.255.224 | 10.2.0.0 | 10.2.0.31 | 10.2.0.33 |
| CX2 | e0 | 30 | 10.3.0.34 | 255.255.255.224 | 10.3.0.0 | 10.3.0.31 | 10.3.0.33 |
| X3 | f2/0.30 | 10 | 10.1.0.65 | 255.255.255.224 | 10.1.0.0 | 10.1.0.63 |  |
|  | f2/0.30 | 20 | 10.2.0.65 | 255.255.255.224 | 10.2.0.0 | 10.2.0.63 |  |
|  | f2/0.30 | 30 | 10.3.0.65 | 255.255.255.224 | 10.3.0.0 | 10.3.0.63 |  |
| AX3 | e0 | 10 | 10.1.0.66 | 255.255.255.224 | 10.1.0.0 | 10.1.0.63 | 10.1.0.65 |
| BX3 | e0 | 20 | 10.2.0.66 | 255.255.255.224 | 10.2.0.0 | 10.2.0.63 | 10.2.0.65 |
| CX3 | e0 | 30 | 10.3.0.66 | 255.255.255.224 | 10.3.0.0 | 10.3.0.63 | 10.3.0.65 |
| X4 | f2/0.30 | 10 | 10.1.0.97 | 255.255.255.224 | 10.1.0.0 | 10.1.0.127 |  |
|  | f2/0.30 | 20 | 10.2.0.97 | 255.255.255.224 | 10.2.0.0 | 10.2.0.127 |  |
|  | f2/0.30 | 30 | 10.3.0.97 | 255.255.255.224 | 10.3.0.0 | 10.3.0.127 |  |
| AX4 | e0 | 10 | 10.1.0.98 | 255.255.255.224 | 10.1.0.0 | 10.1.0.127 | 10.1.0.97 |
| BX4 | e0 | 20 | 10.2.0.98 | 255.255.255.224 | 10.2.0.0 | 10.2.0.127 | 10.2.0.97 |
| CX4 | e0 | 30 | 10.3.0.98 | 255.255.255.224 | 10.3.0.0 | 10.3.0.127 | 10.3.0.97 |
| X6 | f2/0.30 | 10 | 10.1.0.161 | 255.255.255.224 | 10.1.0.0 | 10.1.0.191 |  |
|  | f2/0.30 | 20 | 10.2.0.161 | 255.255.255.224 | 10.2.0.0 | 10.2.0.191 |  |
|  | f2/0.30 | 30 | 10.3.0.161 | 255.255.255.224 | 10.3.0.0 | 10.3.0.191 |  |
| AX | e0 | 10 | 10.1.0.162 | 255.255.255.224 | 10.1.0.0 | 10.1.0.191 | 10.1.0.161 |
| BX6 | e0 | 20 | 10.2.0.162 | 255.255.255.224 | 10.2.0.0 | 10.2.0.191 | 10.2.0.161 |
| CX6 | e0 | 30 | 10.3.0.162 | 255.255.255.224 | 10.3.0.0 | 10.3.0.191 | 10.3.0.161 |
| X7 | f2/0.30 | 10 | 10.1.0.193 | 255.255.255.224 | 10.1.0.0 | 10.1.0.223 |  |
|  | f2/0.30 | 20 | 10.2.0.193 | 255.255.255.224 | 10.2.0.0 | 10.2.0.223 |  |
|  | f2/0.30 | 30 | 10.3.0.193 | 255.255.255.224 | 10.3.0.0 | 10.3.0.223 |  |
| AX7 | e0 | 10 | 10.1.0.194 | 255.255.255.224 | 10.1.0.0 | 10.1.0.223 | 10.1.0.193 |
| BX7 | e0 | 20 | 10.2.0.194 | 255.255.255.224 | 10.2.0.0 | 10.2.0.223 | 10.2.0.193 |
| CX7 | e0 | 30 | 10.3.0.194 | 255.255.255.224 | 10.3.0.0 | 10.3.0.223 | 10.3.0.193 |
| X8Z | f2/0.30 | 10 | 10.1.0.225 | 255.255.255.224 | 10.1.0.0 | 10.1.0.255 |  |
|  | f2/0.30 | 20 | 10.2.0.225 | 255.255.255.224 | 10.2.0.0 | 10.2.0.255 |  |
|  | f2/0.30 | 30 | 10.2.0.225 | 255.255.255.224 | 10.3.0.0 | 10.3.0.255 |  |
| AX8 | e0 | 10 | 10.1.0.226 | 255.255.255.224 | 10.1.0.0 | 10.1.0.255 | 10.1.0.225 |
| BX8 | e0 | 20 | 10.2.0.226 | 255.255.255.224 | 10.2.0.0 | 10.2.0.255 | 10.2.0.225 |
| CX8 | e0 | 30 | 10.2.0.226 | 255.255.255.224 | 10.3.0.0 | 10.3.0.255 | 10.2.0.225 |
| X9Z | f2/0.30 | 10 | 10.1.1.1 | 255.255.255.224 | 10.1.0.0 | 10.1.1.31 |  |
|  | f2/0.30 | 20 | 10.2.1.1 | 255.255.255.224 | 10.2.0.0 | 10.2.1.31 |  |
|  | f2/0.30 | 30 | 10.3.1.1 | 255.255.255.224 | 10.3.0.0 | 10.3.1.31 |  |
|  | f3/0.40 | 40 | 10.4.0.33 | 255.255.255.224 |  | 10.4.1.31 |  |
| AX9 | e0 | 10 | 10.1.1.2 | 255.255.255.224 | 10.1.0.0 | 10.1.1.31 | 10.1.1.1 |
| BX9 | e0 | 20 | 10.2.1.2 | 255.255.255.224 | 10.2.0.0 | 10.2.1.31 | 10.2.1.1 |
| CX9 | e0 | 30 | 10.3.1.2 | 255.255.255.224 | 10.3.0.0 | 10.3.1.31 | 10.3.1.1 |
| Y1 | f2/0.30 | 10 | 10.1.1.33 | 255.255.255.224 | 10.1.0.0 | 10.1.1.63 |  |
|  | f2/0.30 | 20 | 10.2.1.33 | 255.255.255.224 | 10.2.0.0 | 10.2.1.63 |  |
|  | f2/0.30 | 30 | 10.3.1.33 | 255.255.255.224 | 10.3.0.0 | 10.3.1.63 |  |
| AY1 | e0 | 10 | 10.1.1.34 | 255.255.255.224 | 10.1.0.0 | 10.1.1.63 | 10.1.1.33 |
| BY1 | e0 | 20 | 10.2.1.34 | 255.255.255.224 | 10.2.0.0 | 10.2.1.63 | 10.2.1.33 |
| CY1 | e0 | 30 | 10.3.1.34 | 255.255.255.224 | 10.3.0.0 | 10.3.1.63 | 10.3.1.33 |
| Y2 | f2/0.30 | 10 | 10.1.1.65 | 255.255.255.224 | 10.1.0.0 | 10.1.1.95 |  |
|  | f2/0.30 | 20 | 10.2.1.65 | 255.255.255.224 | 10.2.0.0 | 10.2.1.95 |  |
|  | f2/0.30 | 30 | 10.3.1.65 | 255.255.255.224 | 10.3.0.0 | 10.3.1.95 |  |
| AY2 | e0 | 10 | 10.1.1.66 | 255.255.255.224 | 10.1.0.0 | 10.1.1.95 | 10.1.1.65 |
| BY2 | e0 | 20 | 10.2.1.66 | 255.255.255.224 | 10.2.0.0 | 10.2.1.95 | 10.2.1.65 |
| CY2 | e0 | 30 | 10.3.1.66 | 255.255.255.224 | 10.3.0.0 | 10.3.1.95 | 10.3.1.65 |
| Y3 | f2/0.30 | 10 | 10.1.1.97 | 255.255.255.224 | 10.1.0.0 | 10.1.1.127 |  |
|  | f2/0.30 | 20 | 10.2.1.97 | 255.255.255.224 | 10.2.0.0 | 10.2.1.127 |  |
|  | f2/0.30 | 30 | 10.3.1.97 | 255.255.255.224 | 10.3.0.0 | 10.3.1.127 |  |
| AY3 | e0 | 10 | 10.1.1.98 | 255.255.255.224 | 10.1.0.0 | 10.1.1.127 | 10.1.1.97 |
| BY3 | e0 | 20 | 10.2.1.98 | 255.255.255.224 | 10.2.0.0 | 10.2.1.127 | 10.2.1.97 |
| CY3 | e0 | 30 | 10.3.1.98 | 255.255.255.224 | 10.3.0.0 | 10.3.1.127 | 10.3.1.97 |
| Y4 | f2/0.30 | 10 | 10.1.1.129 | 255.255.255.224 | 10.1.0.0 | 10.1.1.159 |  |
|  | f2/0.30 | 20 | 10.2.1.129 | 255.255.255.224 | 10.2.0.0 | 10.2.1.159 |  |
|  | f2/0.30 | 30 | 10.3.1.129 | 255.255.255.224 | 10.3.0.0 | 10.3.1.159 |  |
| AY4 | e0 | 10 | 10.1.1.130 | 255.255.255.224 | 10.1.0.0 | 10.1.1.159 | 10.1.1.129 |
| BY4 | e0 | 20 | 10.2.1.130 | 255.255.255.224 | 10.2.0.0 | 10.2.1.159 | 10.2.1.129 |
| CY4 | e0 | 30 | 10.3.1.130 | 255.255.255.224 | 10.3.0.0 | 10.3.1.159 | 10.3.1.129 |
| Y5 | f2/0.30 | 10 | 10.1.1.161 | 255.255.255.224 | 10.1.0.0 | 10.1.1.191 |  |
|  | f2/0.30 | 20 | 10.2.1.161 | 255.255.255.224 | 10.2.0.0 | 10.2.1.191 |  |
|  | f2/0.30 | 30 | 10.3.1.161 | 255.255.255.224 | 10.3.0.0 | 10.3.1.191 |  |
| AY5 | e0 | 10 | 10.1.1.162 | 255.255.255.224 | 10.1.0.0 | 10.1.1.191 | 10.1.1.161 |
| BY5 | e0 | 20 | 10.2.1.162 | 255.255.255.224 | 10.2.0.0 | 10.2.1.191 | 10.2.1.161 |
| CY5 | e0 | 30 | 10.3.1.162 | 255.255.255.224 | 10.3.0.0 | 10.3.1.191 | 10.3.1.161 |
| Y6 | f2/0.30 | 10 | 10.1.1.193 | 255.255.255.224 | 10.1.0.0 | 10.1.1.223 |  |
|  | f2/0.30 | 20 | 10.2.1.193 | 255.255.255.224 | 10.2.0.0 | 10.2.1.223 |  |
|  | f2/0.30 | 30 | 10.3.1.193 | 255.255.255.224 | 10.3.0.0 | 10.3.1.223 |  |
| AY6 | e0 | 10 | 10.1.1.194 | 255.255.255.224 | 10.1.0.0 | 10.1.1.223 | 10.1.1.193 |
| BY6 | e0 | 20 | 10.2.1.194 | 255.255.255.224 | 10.2.0.0 | 10.2.1.223 | 10.2.1.193 |
| CY6 | e0 | 30 | 10.3.1.194 | 255.255.255.224 | 10.3.0.0 | 10.3.1.223 | 10.3.1.193 |
| Y7 | f2/0.30 | 10 | 10.1.1.225 | 255.255.255.224 | 10.1.0.0 | 10.1.1.255 |  |
|  | f2/0.30 | 20 | 10.2.1.225 | 255.255.255.224 | 10.2.0.0 | 10.2.1.255 |  |
|  | f2/0.30 | 30 | 10.3.1.225 | 255.255.255.224 | 10.3.0.0 | 10.3.1.255 |  |
| AY7 | e0 | 10 | 10.1.1.226 | 255.255.255.224 | 10.1.0.0 | 10.1.1.255 | 10.1.1.225 |
| BY7 | e0 | 20 | 10.2.1.226 | 255.255.255.224 | 10.2.0.0 | 10.2.1.255 | 10.2.1.225 |
| CY7 | e0 | 30 | 10.3.1.226 | 255.255.255.224 | 10.3.0.0 | 10.3.1.255 | 10.3.1.225 |
| Y8 | f2/0.30 | 10 | 10.1.2.1 | 255.255.255.224 | 10.1.0.0 | 10.1.2.31 |  |
|  | f2/0.30 | 20 | 10.2.2.1 | 255.255.255.224 | 10.2.0.0 | 10.2.2.31 |  |
|  | f2/0.30 | 30 | 10.3.2.1 | 255.255.255.224 | 10.3.0.0 | 10.3.2.31 |  |
| AY8 | e0 | 10 | 10.1.2.2 | 255.255.255.224 | 10.1.0.0 | 10.1.2.31 | 10.1.2.1 |
| BY8 | e0 | 20 | 10.2.2.2 | 255.255.255.224 | 10.2.0.0 | 10.2.2.31 | 10.2.2.1 |
| CY8 | e0 | 30 | 10.3.2.2 | 255.255.255.224 | 10.3.0.0 | 10.3.2.31 | 10.3.2.1 |
| Y9Z | f2/0.30 | 10 | 10.1.2.33 | 255.255.255.224 | 10.1.0.0 | 10.1.2.63 |  |
|  | f2/0.30 | 20 | 10.2.2.33 | 255.255.255.224 | 10.2.0.0 | 10.2.2.63 |  |
|  | f2/0.30 | 30 | 10.2.2.33 | 255.255.255.224 | 10.3.0.0 | 10.3.2.63 |  |
| AY9 | e0 | 10 | 10.1.2.34 | 255.255.255.224 | 10.1.0.0 | 10.1.2.63 | 10.1.2.33 |
| BY9 | e0 | 20 | 10.2.2.34 | 255.255.255.224 | 10.2.0.0 | 10.2.2.63 | 10.2.2.33 |
| CY9 | e0 | 30 | 10.2.2.34 | 255.255.255.224 | 10.3.0.0 | 10.3.2.63 | 10.2.2.33 |
| Y10XZ | f2/0.30 | 10 | 10.1.2.65 | 255.255.255.224 | 10.1.0.0 | 10.1.2.95 |  |
|  | f2/0.30 | 20 | 10.2.2.65 | 255.255.255.224 | 10.2.0.0 | 10.2.2.95 |  |
|  | f2/0.30 | 30 | 10.3.2.65 | 255.255.255.224 | 10.3.0.0 | 10.3.2.95 |  |
| AY10 | e0 | 10 | 10.1.2.66 | 255.255.255.224 | 10.1.0.0 | 10.1.2.95 | 10.1.2.65 |
| BY10 | e0 | 20 | 10.2.2.66 | 255.255.255.224 | 10.2.0.0 | 10.2.2.95 | 10.2.2.65 |
| CY10 | e0 | 30 | 10.3.2.66 | 255.255.255.224 | 10.3.0.0 | 10.3.2.95 | 10.3.2.65 |
| Y11X | f2/0.30 | 10 | 10.1.2.97 | 255.255.255.224 | 10.1.0.0 | 10.1.2.127 |  |
|  | f2/0.30 | 20 | 10.2.2.97 | 255.255.255.224 | 10.2.0.0 | 10.2.2.127 |  |
|  | f2/0.30 | 30 | 10.3.2.97 | 255.255.255.224 | 10.3.0.0 | 10.3.2.127 |  |
| AY11 | e0 | 10 | 10.1.2.98 | 255.255.255.224 | 10.1.0.0 | 10.1.2.127 | 10.1.2.97 |
| BY11 | e0 | 20 | 10.2.2.98 | 255.255.255.224 | 10.2.0.0 | 10.2.2.127 | 10.2.2.97 |
| CY11 | e0 | 30 | 10.3.2.98 | 255.255.255.224 | 10.3.0.0 | 10.3.2.127 | 10.3.2.97 |

Diagram 1: Showing Network Topology

Diagram 2: VLAN Switch settings

Diagram 3: Showing RIP database on Router X1

Diagram 4: Showing OSPF/RIPv2 Redistrinution on router Y11X

Diagram 5: Showing Ping Test between PC AX1 on VLAN A to PC CY5 on VLAN C

# Appendix II - References

Cisco Nexus 7000 series NX-OS layer 2 switching configuration guide - configuring layer 2 switching [CISCO NEXUS 7000 series switches]. Cisco. (2018, November 21). https://www.cisco.com/c/en/us/td/docs/switches/datacenter/sw/nx-os/layer2/configuration/guide/b_Cisco_Nexus_7000_Series_NX-OS_Layer_2_Switching_Configuration_Guide/config_layer_2.html

IP Subnet Calculator. Calculator.net. (n.d.). https://www.calculator.net/ip-subnet-calculator.html

Saputra, A. (2018, December 30). Configure redistribution between Rip and OSPF in Cisco IOS router. MustBeGeek. https://www.mustbegeek.com/configure-redistribution-between-rip-and-ospf-in-cisco-ios-router/